Visual Basic Application ProfileElevInc

Required Files

ProfileElevInc.mvba

A Cogo Database (.gpk) file containing the alignment and profile to compute incremental elevations.

Purpose

To write the Station, Region, and Profile Elevation at a defined increment for a station range to an Excel Spreadsheet.

Tool Activation

In the MicroStation Key-in, type in VBA Load ProfileElevInc.  Then type in VBA Run ProfileElevInc.

OR

Assign the key-ins to a function key.  Both key-ins can be assigned to one function key by separating them with a semicolon.  “VBA Load ProfileElevInc;VBA Run ProfileElevInc”

OR

The ProfileElevInc VBA can be accessed in MicroStation by selecting MnDOT Menu > VBA > Surveys and RW> Surveys VBA Toolbox, which will bring up the following interface.

This assumes you have the standard MnDOT configuration for MicroStation where the variable MS_VBASEARCHDIRECTORIES is pointing to S:\Mndot8iStds\DOT_GEOPAK\VBA.

Procedure

Once activated, the dialog below should appear.

- Select the Job, Chain Name, and Profile Name.  (the profile station has to be based on the selected chain)

- Click "Activate Chain/Profile”.  This activates the Chain and Profile in the background and also displays the station limits of the selected profile for aid in entering a station range in the next step.

- Enter a begin station and end station with regions.  Be sure to set the beginning station on an increment that will be set in the next step.

- Enter an increment.  The station will adjust to get back on an even station after going through a station equation.

- Enter a file name to create.  (Will add the .xls extension automatically.)

- Click "Create Excel File".  Note that this button is initially ghosted out until the "Activate Chain/Profile” button is pressed and is ghosted out again after the "Create Excel File" button is clicked.  The same chain/profile may be processed by starting over at step 2 or another chain/profile may be processed by starting over at step 1.

Below is a sample of the Excel file created: