Visual Basic Application SOEtoDAT

Required Files

SOEtoDAT.mvba

An Input SOE file

A Cogo Database (.gpk) file containing the alignment the station/offsets are based on.

Purpose

To convert an ASCII file containing station, offset, and elevations (SOE) to an ASCII (.dat) file which will directly feed into the Build Triangles tool to create the TIN.  This eliminates the time consuming process of storing the points in the gpk since Visual Basic computes the coordinates in the background without opening Cogo.

SOE File Format

- Columns are comma delimited.  Every line must have the same number of columns.  Empty columns of offset/elevation require commas with nothing between them.  (ie: -12,600.57,,,12,600.43)

- The first two columns of each row must be the station and region followed by one or multiple pairs of offsets and elevations.

  - The station “+” is optional.

  - The region is only the number, it can not contain the “r” (ie: “3”, not “r 3”)

Examples:

Station,Region,Offset,Elev,Offset,Elev,Offset,Elev,Offset,Elev…

1+00,1,-12,990.8,0,991.4,12,990.7

500,3,-12,995.2,0,995.8,12,995.3

Tool Activation

In the MicroStation Key-in, type in VBA Load SOEtoDAT.  Then type in VBA Run SOEtoDAT.

OR

Assign the key-ins to a function key.  Both key-ins can be assigned to one function key by separating them with a semicolon.  “VBA Load SOEtoDAT;VBA Run SOEtoDAT”

OR

The SOEtoDAT VBA can be accessed in MicroStation by selecting MnDOT Menu > VBA > Surveys and RW> Surveys VBA Toolbox, which will bring up the following interface.

This assumes you have the standard MnDOT configuration for MicroStation where the variable MS_VBASEARCHDIRECTORIES is pointing to S:\Mndot8iStds\DOT_GEOPAK\VBA.

Procedure

Once activated, the dialog below should appear.

- Select a job.  (Selections are read from active directory.)

- Select a chain.

- Type or select an input SOE file.

- Type an output file name (or select if appending).  The .dat extension will be added automatically.

- Select Create if creating a new .dat file or Append if appending to an existing .dat file because the SOE files were broken into more than one file.

- Click “Convert SOE To .DAT”.

- The Geopak binary .tin file can be created directly from the output .DAT using the Build Triangles tool.  (Note: The best Dissolve Option setting to avoid getting triangulation across the inside of curves is “Sliver” when doing this.)

- The .tin file should be checked using some/all of the following:

  - Check Triangulation

  - Triangle Statistics

  - Loading DTM Features (triangles, break lines, and/or contours) into a 3D file for review